Proyecto: EVA (Eco Village Asuchillo)

Modelo: Macaw

Descripción de Áreas:

| Espacio | Área (m2) |
| --- | --- |
| Sala, desayunador, cocina | 14.99 |
| Habitación planta baja | 8.12 |
| Baño planta baja | 3.45 |
| Habitación planta alta | 8.12 |
| Baño planta baja | 3.64 |
| Lavandería | 1.27 |
| Piso exterior planta baja | 35.75 |
| Balcón planta alta | 20.49 |
| Depósito interno planta baja | 0.84 |
| Escaleras | 5.35 |

Descripciones Generales:

- La cerámica a utilizarse en este proyecto será proporcionada por el propietario, elegidos por el arquitecto e instalado por el contratista.

- Todos los accesorios y artefactos sanitarios serán de la American Standard.

- Toda la madera a utilizarse deberá ser guapinol en el exterior y cedro macho en el interior procurando sea libre de nudos y torceduras.

- Toda la madera a utilizarse debería estar seca y curada; se le aplicara una mano de sellador y dos manos de laca mate. (acabado natural).

- Top de granito – Blanco dallas en cocina y muebles de baño.

- Gabinetes de cocina y muebles flotantes de baño elaborados de melamina antihumedad 18mm.

Para las paredes:

- Pared de bloques de concreto, acabado repellado y fino. Color y tipo de pintura a escoger por diseñador.

- Enchape cerámico a escoger por el arquitecto. En ducha 1.8o de altura, resto del baño 1.50 de altura.

- Pared liviana tipo "Dry Wall”.

Para pisos:

- Porcelanato mate, color y tamaño a escoger por el diseñador.

- Cerámica antiderrapante, color y tamaño a escoger por el diseñador.

Cielo raso:

- Cielo falso interno plano, donde se indique será conforme pendiente, de gypsum regular, estructura a base de perfiles galvanizados, enmasillado y lijado, aplicar 1 mano de sellador base y 2 manos de acabado final. Aplicar pintura color blanco 1000 mate de primera calidad.

- El cielo en alero o baño será de gypsum MR - 1/2" plano, estructura a base de perfiles galvanizados, enmasillado y lijado, aplicar pintura color a seleccionar por el arquitecto, aplicar 1 mano de sellador base y 2 manos de acabado final., color blanco.

- Losa de concreto repello y fino.

- Cielo raso exterior de láminas de PVC con estructuras metálicas internas. Color simulando madera recomendado por el diseñador.

Puertas y ventanas:

- Herraje y cerradura para puerta metálica exterior serán metálicas de primera calidad acabado inoxidable, marca etc.

- Herraje y cerradura para puerta principal serán metálicas, acabado acero inoxidable, marca etc.

- Herraje y cerradura para puertas en dormitorios serán metálicas, acabado acero inoxidable, marca etc.

- Herraje y cerradura para puertas en baños y bodega, serán metálicas acabado acero inoxidable, marca etc.

- Herraje y cerradura para puerta de corrediza en closet, serán metálicas de primera calidad acabado satinado, marca (Stanley, Best Value o Yale). Instalar riel metálico con rodillo sobre dintel, 1 jaladera tipo pocket, estilo lineal moderno.

- Los marcos se instalarán a plomo y a la línea en los correspondientes huecos y atornilladas a espiche de plomo previamente empotrados en las paredes.

- Los acabados y las cerraduras a usarse deberán repetirse en las bisagra y resto de accesorios que llevaran las puertas.

- Toda la madera a utilizarse en caso de que se requiera, deberá ser caoba, cedro real o pochote, procurando sea libre de nudos y torceduras.

- Toda la madera a usarse deberá estar seca y curada, se le aplicará una mano de sellador y dos manos de laca mate (acabado natural).

- Los acabados tanto en madera como en accesorios de ventanas en caso de usarse deberán ser iguales a los de puertas.

Luminaria:

- Se utilizará luminaria tipo LED en toda la casa.

- Toda la instalación será debidamente polarizada de acuerdo con el código.

- El calibre mínimo del conductor será #12 AWG con aislamiento termoplástico THHN para 600 voltios. El diámetro mínimo de tubería será de ½”.

- Se deberá aterrizar todos los sistemas eléctricos según se indique en el diagrama unifilar en los planos. La impedancia a tierra no deberá exceder de 5 (cinco) ohmios.

- Toda la caja de registro metálica o plástica que este expuesta a la intemperie, deberá ser del tipo especial para intemperie nema 3r. Toda caja de salida para luminarias y dispositivos de uso general será de 4” x 4” y 1 ½” de profundidad, deberán estar provistas con tapa de repello con un levante no menor de ¼”. Las tapas de repello deberán quedar con un repello final o fino del acabado arquitectónico. Las cajas de salida para dispositivos especiales no serán menores de 5” x 5” y 2” de profundidad, deberán estar provistas con tapa ciega.

- Los apagadores y toma corrientes serán colocados a una altura uniforme como regla general.

- Todas las medidas se entienden del nivel de piso terminado a los centros de las cajas de salida.

- Toda canalización para el sistema eléctrico excepto donde se especifique o contrario será PVC cédula h con accesorios del mismo material, como uniones, codos radios largos, conectores (adaptadores machos), etc.

- Todas las tierras del sistema deberán conectarse a la tierra general del módulo de medición.

- Los interruptores termomagnéticos (breakers) de los circuitos de tomacorrientes en cuartos de baños, garajes, áreas exteriores, cocina y lavandería, serán del tipo "falla a tierra GFCI".

Sistemas de agua potable y residual:

- El sistema de aguas negras y ventilación será de PVC sdr-41 con accesorios tipo DWV. No se permitirá el calentamiento de los extremos de los tubos para ensancharlos y usarlo como acople entre tubos, úsense los acoples o camisas hechas por el fabricante.

- El sistema de agua potable será de plástico PVC SDR-13.5 para las tuberías de 13mm∅, SDR-17 para 18mm∅, los accesorios serán para presión.  Cuando en las líneas de tubería de PVC se deriven ramales expuestos a la intemperie o al maltrato físico, estos se efectuarán con tubería de hierro galvanizado cedula 40 y accesorios de hierro galvanizado.

- Cada casa se entrega con el medidor de agua potable instalado.

Sistema solar:

- Sistema solar off grid con capacidad instalada de 7.36 KWp, potencia nominal 8Kw conectado con los siguientes compo-nentes: 1 Inversor Hibrido Growatt SPF 8000T DVM-MPV, 16 Paneles Solares PEIMAR OR6H460M 460W, 1 Combiner BOX, 3 Baterías de Litio Growatt HOPE 4.8L-C1 de 100 Ah, 1 Interruptor de seguridad de 250A para Banco de baterías, 1 Transferencia Manual de 3 posiciones y 1 Datalogger.

Project: EVA (Eco Village Asuchillo)

Model No.: Macaw

Description of Areas:

| Space | Area (m2) |
| --- | --- |
| Living room, breakfast area, kitchen | 14.99 |
| Ground floor room | 8.12 |
| Ground floor bathroom | 3.45 |
| Upstairs room | 8.12 |
| Ground floor bathroom | 3.64 |
| Laundry | 1.27 |
| Ground floor exterior floor | 35.75 |
| Balcony top floor | 20.49 |
| Internal warehouse on the ground floor | 0.84 |
| Stairs | 5.35 |

General Descriptions:

         The ceramics to be used in this project will be provided by the owner, chosen by the architect and installed by the contractor.

         All fixtures and sanitary fixtures will be of the American Standard.

         All the wood to be used must be guapinol on the outside and male cedar on the inside, trying to be free of knots and kinks.

         All wood to be used should be dried and cured; One coat of sealant and two coats of matte lacquer will be applied. (natural finish).

         Granite Top – White dallas in kitchen and bathroom furniture.

         Kitchen cabinets and floating bathroom furniture made of 18mm anti-humidity melamine.

For the walls:

         Concrete block wall, plastered and fine finish. Color and type of paint to choose per designer.

         Ceramic veneer to be chosen by the architect. In shower 1.8° high, rest of the bathroom 1.50° high.

         Lightweight "Dry Wall" type wall.

For floors:

         Matte porcelain tile, color and size to be chosen by the designer.

         Non-slip ceramic, color and size to be chosen by the designer.

Ceiling:

         Flat internal false ceiling, where indicated will be in accordance with the slope, of regular gypsum, structure based on galvanized profiles, caulking and sanding, apply 1 coat of base sealant and 2 coats of final finishing. Apply 1000 matte white paint of first quality.

         The ceiling in eaves or bathroom will be gypsum MR - 1/2" flat, structure based on galvanized profiles, caulking and sanding, apply color paint to be selected by the architect, apply 1 coat of base sealant and 2 coats of final finish., white color.

         Plastered and thin concrete slab.

         Exterior ceiling of PVC sheets with internal metal structures. Color simulating wood recommended by the designer.

Doors and windows:

         Hardware and lock for exterior metal door will be top quality metal finish, stainless finish, brand etc.

         Hardware and lock for the main door will be metallic, stainless steel finish, brand etc.

         Hardware and lock for bedroom doors will be metallic, stainless steel finish, brand etc.

         Hardware and lock for doors in bathrooms and cellar, they will be metal finished stainless steel, brand etc.

         Hardware and lock for sliding doors in closets, will be top quality metal satin finish, brand (Stanley, Best Value or Yale). Install metal rail with roller on lintel, 1 pocket handle, modern linear style.

         The frames will be installed with lead and line in the corresponding openings and screwed to lead spike previously embedded in the walls.

         The finishes and locks to be used must be repeated on the hinges and other accessories that will be on the doors.

         All the wood to be used, if required, must be mahogany, royal cedar or pochote, ensuring that it is free of knots and kinks.

         All the wood to be used must be dry and cured, one coat of sealant and two coats of matte lacquer (natural finish) will be applied.

         The finishes on both wood and window accessories, if used, must be the same as those on doors.

Luminary:

         LED lighting will be used throughout the house.

         The entire installation will be properly polarized according to the code.

         The minimum conductor gauge will be #12 AWG with THHN thermoplastic insulation for 600 volts. The minimum pipe diameter shall be 1/2".

         All electrical systems must be landed as indicated in the one-line diagram on the plans. The impedance to ground shall not exceed 5 (five) ohms.

         All metal or plastic manhole that is exposed to the weather must be of the special type for NEMA 3R weathering. All outlet boxes for luminaires and general purpose devices shall be 4" x 4" and 1 1/2" deep, and shall be provided with a plaster lid with a lift of not less than 1/4". The plaster lids must be left with a final or fine plaster of the architectural finish. Outlet boxes for special devices shall not be less than 5" x 5" and 2" deep, and shall be provided with a blind lid.

         Switches and outlets should be placed at a uniform height as a general rule.

         All measurements are understood from the finished floor level to the centers of the outlet boxes.

         All conduits for the electrical system, except where specified or otherwise, shall be schedule h PVC with accessories of the same material, such as joints, elbows, long radii, connectors (male adapters), etc.

         All soils in the system shall be connected to the general ground of the measurement module.

         Thermomagnetic circuit breakers in bathrooms, garages, outdoor areas, kitchens, and laundry rooms will be of the "GFCI ground fault" type.

Drinking and waste water systems:

         The sewage and ventilation system will be made of PVC sdr-41 with DWV type accessories. The heating of the ends of the tubes to widen them and use it as a coupling between tubes will not be allowed, use couplings or jackets made by the manufacturer.

         The drinking water system will be made of PVC plastic SDR-13.5 for the 13mm∅ pipes, SDR-17 for 18mm∅, the fittings will befor pressure. When branches exposed to the weather or physical abuse are derived from PVC pipe lines, these shall be carried out with galvanized iron pipe schedule 40 and galvanized iron fittings.

         Each house is delivered with the drinking water meter installed.

Solar system:

         Off grid solar system with an installed capacity of 7.36 KWp, nominal power 8Kw connected with the following components: 1 Growatt SPF 8000T DVM-MPV Hybrid Inverter, 16 PEIMAR OR6H460M 460W Solar Panels, 1 Combiner BOX, 3 Growatt HOPE 4.8L-C1 100 Ah Lithium Batteries, 1 250A safety switch for battery bank, 1 3-position Manual Transfer and 1 Datalogger.